WHITE PAPER

Efficient Coding: The EFMW System

A bounded engineering specification for an O(1) residual-EWMA early-warning kernel, including what the public EFMW-EXP-001 monitor actually computes and how to implement it without surplus state.

Classification: engineering implementation note. Not a physics paper. Not a claim of a new detection law.

Date: 18 September 2026

# 1. Abstract

This paper specifies the smallest correct program for the monitor published as the EFMW-EXP-001 “recursive coherence” score. That program is a two-sided exponentially weighted moving average of an ARX prediction residual. It is not a field theory and it does not require an extra computational layer named EFMW.

On a slow mean-bias fault in a scalar control loop, |EWMA(r)| is the right cheap statistic. EWMA(|r|) is the wrong cheap statistic. Both cost four arithmetic operations per sample. Efficient coding therefore means: implement the signed-residual chart, studentize it, debounce it, freeze the threshold on healthy data, and refuse every additional state variable.

The deliverable is a kernel: five floats of state, a C inner loop, and a calibration rule. Claims beyond that kernel are out of scope.

# 2. Scope and non-claims

In scope: scalar measured output y_t, known command u_t, one-step predictor, residual, EWMA, two-sided threshold, optional running scale, persistence rule, embedded implementation.

Out of scope: Einstein–Maxwell–Feynman geometry, information tensors, observer-participatory ontology, general theories of emergence, publication of a new control chart, and any statement that this kernel “is the EFMW field.”

Public EXP-001 compared two scores on the same residual. The score labeled EFMW is |EWMA(r)| with λ = 0.97. The score labeled baseline is EWMA(|r|) with λ = 0.90. Both use a three-sample persistence rule and separately calibrated thresholds. That is the entire published increment.

# 3. The system that is actually coded

## 3.1 Residual

Let the one-step predictor be the given ARX form used in EXP-001 and in the blind packet:

r_t = y_t − (a y_{t−1} + u_t),    a = 0.82 unless identified from healthy data.

This subtracts the part of the output already explained by the last measurement and the known command. What remains is an innovation. A slow plant-pole or gain drift appears, on this plant class, as a persistent mean in r_t.

## 3.2 Memory

m_t = λ m_{t−1} + (1 − λ) r_t,    λ ∈ [0.90, 0.97] for slow drift.

This is exponential smoothing. It is not a second physical memory. Time constant is about 1/(1−λ) samples (λ = 0.97 ⇒ ~33 steps).

## 3.3 Contrast / decision

C_t = |m_t|. Alarm if C_t (or C_t / ŝ_t) exceeds τ for K consecutive samples. Default K = 3.

Absolute value is two one-sided charts packed into one test. It adds no information beyond a pair of limits on m_t.

## 3.4 What EXP-001 added relative to its own baseline

| Item | Baseline in EXP-001 | Score labeled EFMW | Engineering status |
| --- | --- | --- | --- |
| Residual | same r_t, a=0.82 | same r_t, a=0.82 | identical |
| Smoother | EWMA of ¦r¦, λ=0.90 | EWMA of r, then ¦m¦, λ=0.97 | mean-shift vs energy-like |
| Decision | 3-in-a-row vs own τ | 3-in-a-row vs own τ | run rule |
| State | one float b_t | one float m_t | same order |
| FLOPs / step | ~4 | ~4 | no compute win |

The published confirmatory compute ratio was 1.117 in the direction of slightly more work for the EFMW-labeled score. There is no efficiency gain in wall-clock or asymptotic cost. The only “efficiency” is that both charts are already O(1), and |EWMA(r)| extracts more detection per flop on a mean fault than EWMA(|r|).

# 4. Why this is the efficient program

A controller already has y and u. After the control law computes u_t, the monitor is four fused-multiply-adds. It does not allocate, does not touch a buffer of residuals, and does not call a model server.

Heavier alternatives at the same job — windowed CUSUM with a long buffer, a batch likelihood ratio, an online learner — spend memory and branches the MCU does not have. A well-tuned two-sided CUSUM on the same r_t is in the same flop class; choose it if you already have that code. Do not invent a third filter to justify a name.

Efficiency is therefore a coding discipline:

- One residual, computed from the live y and the live u.

- One EWMA on the signed residual, not on |r| and not on r² as the primary score.

- Optional second EWMA on r² only to studentize the score (scale, not a second detector).

- Integer persist counter. No queues.

- Threshold from a healthy quantile of the same score the kernel emits.

- Recalibrate when sensors or noise scale change. Do not freeze τ across a distribution shift and call the false alarms a field effect.

# 5. Ideal operating context

| Fits | Does not fit |
| --- | --- |
| Slow persistent mean in r_t | Oscillatory / sign-changing precursor |
| y and u already sampled | u unknown or badly delayed |
| Healthy data for τ | No in-control record |
| MCU / PLC / ISR budget | Need isolation of which component failed |
| Stationary noise after calibration | Sudden mean or variance shift of the noise |

Independent synthetic checks on a closed-loop ARX plant with gradual pole drift found |EWMA(r)| above raw |r| and above EWMA(r²) at matched pre-onset false-positive rate, with a high miss rate at that operating point and breakage under frozen-τ noise-mean and variance shifts. That is the classical EWMA profile. Code should expose studentization precisely because the frozen-|m| chart is brittle.

# 6. Reference kernel

Python state machine (logic identical to the C kernel in residual_ewma_monitor.py):

r  = y - (a * y_prev + u)

m  = lam * m + (1 - lam) * r

s2 = scale_lam * s2 + (1 - scale_lam) * r * r

score = abs(m) / max(sqrt(s2), min_scale)   # or abs(m) if not studentized

y_prev = y

if n >= burn_in:

run = run + 1 if score > thr else 0

alarm = run >= persist

State vector: {y_prev, m, s2, n, run, alarmed}. No heap.

C ISR-shaped inner loop (sqrt once per sample is acceptable on any MCU that already runs a PID; drop studentize if the FPU is absent and keep a frozen raw threshold):

double r = y - (s->a * s->y_prev + u);

s->m = s->lam * s->m + (1.0 - s->lam) * r;

s->s2 = s->scale_lam * s->s2 + (1.0 - s->scale_lam) * r * r;

s->y_prev = y;

double score = fabs(s->m);

if (s->studentize) score /= fmax(sqrt(s->s2), s->min_scale);

s->run = (score > s->thr) ? s->run + 1 : 0;

return s->run >= s->persist;

# 7. Calibration protocol

- Collect healthy (y, u) under the same sampling interval as production.

- Run the kernel with the production (a, λ, studentize) settings. Discard burn-in.

- Set τ to a high quantile of the score (0.90–0.99 depending on tolerated false-alarm rate).

- If comparing two charts, match false-alarm rate on the same healthy set. Do not compare two independently frozen τ values and treat lead-time gap as a discovery.

- Identify a from healthy data when possible. The published 0.82 is a frozen example, not a universal constant.

# 8. Integration pattern

Place the call after the control law, in the same tick:

u = control_law(y, ref);

apply_actuator(u);

if (rewma_step(&mon, y, u, &score)) raise_warning(score);

Do not run the monitor in a cloud worker “over time” to harvest an efficiency narrative. The only time that matters is the controller tick.

# 9. Failure modes the code must document

| Fault | What the kernel does | What to do |
| --- | --- | --- |
| Mean shift in noise | m tracks the new mean; frozen τ false-alarms | Recalibrate; studentize does not fix a mean jump |
| Variance inflation | raw ¦m¦ false-alarms | Studentize or widen τ |
| Oscillatory residual | EWMA cancels; late or missed | Different chart (window, CUSUM reset) |
| Wrong a, no recalibration of τ | standing bias absorbed only if τ was fit after the mismatch | Fit a or fit τ, not neither |
| Command not in the residual | u-driven motion looks like a fault | Always subtract measured u |

# 10. What remains after prior-art subtraction

Roberts (1959) EWMA charts; residual charts for autocorrelated and controlled processes; Montgomery–Mastrangelo one-step EWMA predictors; residual-EWMA fault detection in process plants. After that subtraction, the coded system is an implementation of |EWMA(r)| on an ARX residual with a run rule and optional scale EWMA.

Efficient coding does not create a remainder called EFMW. It creates a 20-line ISR that a technician can audit.

# 11. Claims permitted in accompanying software

Permitted: “O(1) residual-EWMA mean-shift monitor for loops that already measure y and u.” “Studentized two-sided EWMA of an ARX innovation, persist-K alarm.”

Not permitted in this specification: “EFMW field detector,” “coherence waveform,” “new early-warning physics,” “compute advantage over residual-EWMA,” or “validated beyond a scalar synthetic drift.” If a product name keeps the letters EFMW, the README must state that the executable is residual-EWMA.

# 12. Recommended next implementation work

- Drop this kernel on one real loop with an independent degradation label.

- Report ROC at matched false-alarm rate against CUSUM on the same residual.

- Do not add states until that comparison loses.

# 13. Conclusion

The efficient program is the small one. Signed residual, one EWMA, absolute value or two limits, optional scale, persist counter, healthy quantile. That is the EFMW-EXP-001 score, stripped of vocabulary. Coding to advantage the efficiency means keeping that surface area and putting it in the control tick — not building a system around the name.

# Appendix A. Reference files

residual_ewma_monitor.py — Python class, calibration helper, C kernel string.

Public EXP-001 monitors.py — residuals(); baseline_score = EWMA(|r|); efmw_score = |EWMA(r)|.

# Appendix B. Independent numerical note

A separate confirmatory simulation (N_test = 120, T = 800, onset = 400, matched pre-onset FPR ≈ 0.08–0.10) produced detection rates of 0.308 for |EWMA(r)| versus 0.067 for |r| and 0.075 for EWMA(r²), with the candidate identical to a residual-EWMA baseline that used the same signed smoother. Those figures characterize one plant class. They are not product performance specifications.